SEN-36005Configuration

- Connect sensor to CAN adapter

Red: Power: +7V -- +16V

Black: Ground

Yellow: CAN-High

Green: CAN-Low

- Run Komodo GUI

  - Connect port

  - Set bitrate or auto bitrate if the sensor is running and should change to 1000 kHz

  - Choose channel A or B

  - Run

- Get Hardware Serial Number

Find the heartbeat message(1Hz) of message ID 0x1F0B01FF(FF is 00 for default, and 01 in this example)

The Hardware Serial Number is byte 1-3, in this example [1B 1B 00]

- Send the message to the sensor to rename the device ID

CAN ID: 0x1f0b03ff

DLC(data length):7

Extended ID(29bit): yes

Data: 0C 1B 1B 00 10 01 03    or now some come with manufacturer number 0x111 soData: 0C 1B 1B 00 11 01 03

Change to your hardware serial number

Change to your new device ID

After you send the message, the sensor message ID end with 03(your new device name)

- Ranging, sample time configuration (optional, 1.4m short range by default)

Send a message, similar to step 4

CAN ID: 0x060b04ss(your device ID)

DLC(data length):8

Extended ID(29bit): yes

To Send 50ms is 32Hex

Data:  00 32 00 00 00 00 0F 0F

Range mode: 00: short mode 1.4m(default)

01: medium mode

02: long mode 4m (only in dark)

(The longer mode used, the darker environment should be)

Sample time(ms): 10-1000, (default 10ms)

DataSheet says minimum sample rate is 24 but it is actually 10ms.